Codebook for Criteria Air Pollutant Monitoring Dataset

[Please wait until the February 2026 to upload this document to GRIIDC]

Project Title: Community-driven Geospatial Approach to Improving Public Health Data Systems in Beaumont-Port Arthur, Texas

Responsible Personnel: Dr. Tianjun Lu (tianjunlu@uky.edu)

Version: v1.0

Date: [Date of submission]

Overview

This dataset contains de-identified, hourly air quality measurements collected using TSI BlueSky monitors deployed in Beaumont and Port Arthur, Texas. Each entry corresponds to a monitor-hour record and includes common air pollutants (e.g., PM2.5, NO2, O3), meteorological data, and the Census Tract of the monitoring location. All coordinates have been removed to protect participant and household privacy.

Codebook Table

| Variable | Type | Description |
| --- | --- | --- |
| City | String | City of the monitoring site (Beaumont or Port Arthur) |
| MonitorID | String | Unique 13-digit identifier of the TSI BlueSky monitor |
| HouseID | String | De-identified unique ID for the household hosting the monitor (e.g., H01) |
| CensusTract | String | 11-digit 2020 Census Tract ID for the monitor location |
| Hour | DateTime | UTC timestamp marking the start of each hourly interval |
| PM2.5_adj | Float | Adjusted PM2.5 concentration (μg/m3) using collocated correction equation |
| NO2 | Float | Hourly mean nitrogen dioxide concentration (ppb) |
| SO2 | Float | Hourly mean sulfur dioxide concentration (ppb) |
| PM10 | Float | Hourly mean PM10 concentration (μg/m3) |
| CO | Float | Hourly mean carbon monoxide concentration (ppm) |
| O3 | Float | Hourly mean ozone concentration (ppb) |
| CO2 | Float | Hourly mean carbon dioxide concentration (ppm) |
| Temp | Float | Hourly mean ambient temperature (°C) |
| RH | Float | Hourly mean relative humidity (%) |
| Time_Texas | String | Local time (CST/CDT) |

Notes on Data Cleaning and Quality Control

- Raw sensor outliers were flagged and adjusted or removed using automated QA/QC checks: Flatline detection, negative/physically implausible values, sudden spikes not corroborated by PM10 or NO2, Temperature and RH sanity limits, etc.

- Coordinates were converted to 2020 Census Tract IDs using TIGER/Line shapefiles for de-identification.

- Timestamps were stored in UTC (Hour) and Texas local time (Time_Texas) for transparency.

Suggested Citation

Lu et al. (2026). Hourly Ambient Air Quality Monitoring in Beaumont and Port Arthur, Texas (Nov 2024–June 2025) [Data set]. GRIIDC. https://doi.org/[TO_BE_FILLED_AFTER_SUBMISSION]

Data Use and Acknowledgment

Researchers who wish to use this dataset for publications, presentations, or derivative works are kindly asked to contact the responsible investigator, Dr. Tianjun Lu (tianjunlu@uky.edu), prior to use. Co-authorship is expected for substantial reuse, analysis, or publication of this dataset. Please ensure proper acknowledgment of this data source in all outputs, and cite the dataset using the suggested citation format provided above.